---

alwaysApply: false

---

# Vitis HLS Code Guidelines

# Dataflow

- Always use #pragma HLS DATAFLOW at the top scope to enable task-level parallelism across independent functions or loops.

- Ensure producers and consumers are connected with minimal cross-dependencies to achieve true parallel execution.

- Always exchange data between dataflow processes using handshake channels like hls::stream or ping-pong buffers.

- Declare hls::stream<T> with #pragma HLS STREAM depth=<N>, ensure at least depth=1 to avoid deadlock, and prefer moderate depth (8–16) if unsure.

- Map top-level stream ports to AXI-Stream or ap_fifo interfaces so they become true FIFO signals in RTL.

- Always use blocking reads and writes on hls::stream channels in dataflow regions to ensure natural synchronization through backpressure.

- For every stream read in one task, ensure there is a matching write in another task, and vice versa.

- Avoid unbroken dependency cycles in the dataflow graph.

- If a feedback path is required, insert a FIFO with sufficient depth and at least one element of delay to break the cycle.

- Do not use pure feedback between dataflow processes unless a buffer is present.

- If simulation shows deadlock or stalls, increase FIFO depth or refactor to remove the cycle.

- Structure dataflow tasks to overlap computation, input reading, and output writing.

- Implement separate functions for input, compute, and output, and connect them with hls::stream channels.

- Pipeline each stage internally with #pragma HLS PIPELINE so multiple data chunks are processed concurrently.

- Ensure the design sustains one new data item per cycle across the full accelerator pipeline.

- Do not communicate between dataflow processes using global or static variables.

- Never share arrays or variables across tasks without handshake channels.

- Each dataflow process must only operate on local variables and its stream I/O.

- If multiple tasks need a common array, convert it into a stream or use a double-buffer scheme instead of shared access.

- Do not inline functions that should run as separate tasks in a dataflow region.

- Keep concurrent tasks as distinct functions or loops with their own stream interfaces.

- Do not call the same function instance twice inside dataflow unless they are duplicated or templated to create separate hardware.

- Do not reuse the same function instance for multiple tasks in a dataflow region, because it forces sequential execution and breaks parallelism.

- Do not leave hls::stream FIFO depths at default values.

- Always set FIFO depth large enough to cover consumer pipeline latency or input bursts.

- Too shallow a FIFO can cause stalls or deadlock; increase depth to maintain throughput.

- If unsure, use a moderate depth and adjust after performance analysis.

- Do not ignore HLS tool warnings about potential deadlock in dataflow regions.

- Deadlock usually indicates a cycle with insufficient buffering or a dependency issue.

- Fix deadlocks by inserting a FIFO of depth >= 1 in the cycle or restructuring to remove zero-latency dependencies.

- If a producer and consumer wait on each other with an empty FIFO, add extra buffering or decouple the read/write sequence.

- Do not apply #pragma HLS DATAFLOW to tasks with strict sequential dependencies.

- If Task B depends on the full completion of Task A, keep them sequential or restructure the algorithm.

- Using dataflow on dependent tasks adds complexity and FIFOs without performance gain.

# Pipeline

- Apply #pragma HLS PIPELINE to innermost or performance-critical loops.

- Always target Initiation Interval (II) = 1 so the loop accepts new data each cycle.

- Place the PIPELINE pragma at the loop definition, or at the function definition to pipeline the entire function.

- Use higher II values only if resource limits or dependencies prevent II=1.

- Resolve loop-carried dependencies to achieve the target Initiation Interval (II).

- Reorganize computations or use techniques like double buffering to remove true dependencies.

- If a variable is reused across iterations, separate it per iteration (e.g. index by loop counter) or declare it static if it represents persistent state.

- Only apply #pragma HLS DEPENDENCE ... false for dependencies you are certain are false.

- Use the II=<N> option in #pragma HLS PIPELINE to specify the target initiation interval.

- HLS defaults to II=1; set higher II only if dependencies or resource limits require it.

- Use the rewind option for infinite or streaming loops so pipeline registers are reused across iterations.

- Pipelined functions inherently auto-rewind and can process streaming data continuously without termination.

- Use #pragma HLS PIPELINE ... enable_flush (or flushable/free_running style) for loops that may pause due to lack of input or feedback paths.

- A flushable pipeline continues shifting data even without new inputs, preventing stalls and deadlocks.

- Apply flushable or free_running only when needed, since they may increase resource usage or raise II.

- Combine pipelining with loop optimizations like unroll, flatten, and merge for best performance.

- Unroll small loops by a factor (e.g. 2 or 4) to expose parallelism, and ensure enough memory ports with #pragma HLS ARRAY_PARTITION.

- Use #pragma HLS LOOP_FLATTEN to collapse nested loops into one larger pipeline when nested structure is unnecessary.

- Apply #pragma HLS LOOP_MERGE to combine consecutive independent loops and reduce pipeline restart overhead.

- Use these transformations judiciously and always verify functional correctness.

- Use #pragma HLS LOOP_TRIPCOUNT on loops without compile-time constant bounds to guide optimization.

- Set sensible min and max tripcount values based on expected iteration range.

- Tripcount is a hint, not a hard limit; it helps with scheduling, FIFO sizing, and memory bursts.

- For loops with data-dependent exits, provide an upper bound via tripcount and use assert() to enforce assumptions.

- Do not ignore loops with II > 1; always identify the cause.

- Common causes include memory port conflicts, computational dependencies, or limited hardware resources.

- Use #pragma HLS ARRAY_PARTITION to create multiple memory banks and allow concurrent accesses.

- Increase resource budgets or modify the algorithm to remove bottlenecks when possible.

- Accept II > 1 only as a deliberate trade-off (e.g. saving area); otherwise always strive for II=1.

- Do not write loop code with inherent serialization that prevents pipelining.

- Avoid back-to-back dependent operations on the same variable across iterations.

- If each iteration depends on the previous, HLS will raise II and lose concurrency.

- Restructure algorithms using techniques like arrays or indexed storage to allow independent parallel iterations.

- Do not use recursion in pipelined functions or loops; hardware cannot support it.

- Avoid unbounded while or for loops that lack a compile-time termination.

- If a continuous process is required, use a static steady-state loop with a fixed max tripcount.

- For true free-running pipelines, use ap_ctrl_none to design always-running processes explicitly.

- Do not apply #pragma HLS PIPELINE at the function level unless the design is meant to be a continuous streaming process.

- Function-level pipelining treats the function as auto-rewind, which is only correct for always-running streaming kernels.

- For functions that must compute a result and return, pipeline the internal loops instead of the whole function.

- Prefer loop-level pipelining to achieve performance goals; avoid function-level pipeline unless explicitly required.

- Do not write pipelined loops with multiple exit points or break conditions.

- Multiple exits complicate control logic and often increase II or reduce performance.

- If early termination is needed, restructure the loop to have a single exit or use a flag checked at the loop head.

- For data-dependent termination, consider using a flushable pipeline to avoid stalls.

- Do not ignore resource conflicts when pipelining; parallel iterations may compete for the same hardware.

- If two iterations access the same memory port or math unit, the pipeline will stall and II will increase.

- Avoid conflicts by using dual-port BRAMs, adding memory banks, or applying #pragma HLS ARRAY_PARTITION.

- Plan memory and operator resources explicitly for parallel access; do not rely on the tool to resolve conflicts automatically.

- Do not fully unroll large loops without verifying resource budgets.

- Be aware that unrolling replicates hardware, which increases area and may reduce clock frequency.

- Do not fully unroll large loops unless resources and timing can support it.

- Prefer pipelining with II=1 to achieve one iteration per cycle without full replication.

- Use partial unrolling only when needed to expose parallelism or remove bottlenecks.

- Choose an unroll factor that balances performance needs with available resources.

# Hierarchical Design

- Partition the design into multiple functions, each handling a distinct stage of computation.

- Encapsulate one primary loop or kernel per function to optimize it in isolation.

- Use hierarchical partitioning to improve clarity and enable concurrency.

- Place independent functions under #pragma HLS DATAFLOW to run them in parallel and increase throughput.

- Connect functions via arguments: use pointers/arrays for large data and hls::stream for streaming data.

- In a top-level function with multiple sub-functions, apply #pragma HLS DATAFLOW and connect them with static hls::stream<T>.

- Always assign FIFO depth with #pragma HLS STREAM depth=<N> to avoid deadlock and control buffering.

- At the top function, assign appropriate interface pragmas: axis for streams, m_axi for memory, s_axilite for config, and ap_ctrl_hs or ap_ctrl_none for control.

- Use #pragma HLS INLINE for small utility functions or those called only once to remove call overhead and enable better optimization.

- Use #pragma HLS INLINE off for functions that should remain distinct hardware blocks.

- Do not inline functions intended to run in parallel under #pragma HLS DATAFLOW.

- Keep independent tasks as separate functions to preserve concurrency and avoid resource sharing conflicts.

- Remember that by default, multiple calls to the same non-inlined function reuse one hardware instance sequentially.

- If you need parallel hardware instances, call the function from separate dataflow processes.

- Alternatively, duplicate the function code with different names or use C++ templates to force distinct hardware blocks.

- Each unique function (by name or template parameters) generates one hardware block; plan replication explicitly.

- Use #pragma HLS INTERFACE ap_ctrl_none port=return for internal sub-functions that run continuously inside dataflow.

- ap_ctrl_none removes start/stop handshaking and treats the sub-function as a free-running process.

- Only apply ap_ctrl_none to internal stages; the top-level kernel should keep a control handshake (e.g. s_axilite).

- Ensure ap_ctrl_none functions loop or process continuously and do not stall or reset unexpectedly.

- Balance latency between parallel branches that reconverge to avoid stalls or deadlock.

- If one branch is faster, add FIFO depth to buffer its outputs until the slower branch catches up.

- Optionally add small delays in the faster branch, though adjusting FIFO sizes is preferred.

- Ensure that after startup, all parallel branches can produce one result per cycle without starving or overflowing.

- Do not implement the entire algorithm as one monolithic function with deeply nested loops.

- Break the design into logical sub-functions, such as preprocessing, core computation, and postprocessing.

- Modular designs improve readability, simplify debugging, and allow individual optimization.

- Use #pragma HLS DATAFLOW on independent functions to run them in parallel for higher throughput.

- Do not use unsynthesizable constructs in any HLS module.

- Prohibited items include dynamic memory allocation (malloc, new), deallocation (free, delete), recursion, and virtual functions.

- Each function must map to a finite hardware structure with static resources.

- Use static arrays or objects for fixed memory, and loops/unrolling instead of recursion for repeated behavior.

- Do not rely on default interface synthesis for top-level functions; always specify interface pragmas explicitly.

- Use #pragma HLS INTERFACE m_axi for arrays/pointers to external memory, s_axilite for scalar parameters, and axis for streaming ports.

- Ensure the top-level function includes ap_ctrl_hs (AXI4-Lite control) unless ap_ctrl_none is intentionally chosen.

- Proper interface pragmas are required for correct integration and optimal performance.

- Do not use static or global variables across multiple functions unless intentional.

- Sharing global or static state connects functions in hardware and may introduce dependencies or timing issues.

- If independent state is required, pass data through function arguments or keep state self-contained in each function.

- For true shared state (e.g., configuration), be aware it becomes a fixed hardware register or memory, not a port.

- Do not introduce feedback loops in the system-level dataflow without careful planning.

- Feedback complicates scheduling and often prevents effective task-level parallelism.

- If feedback is required, insert FIFO buffers with sufficient depth and consider using #pragma HLS DATAFLOW carefully.

- Ensure feedback loops cannot deadlock: provide initialization, break conditions, or an initial token in the FIFO.

- Prefer acyclic (feed-forward) dataflow designs whenever possible.

# Data Types

- Prefer ap_int<W> or ap_uint<W> over native int/long when exact bit widths are needed.

- Match bit width to the value range for efficient resource usage (e.g., ap_uint<10> for a 10-bit counter).

- Use the smallest width that covers the range, but allow extra bits if carry/overflow is possible.

- Remember that ap_int/ap_uint arithmetic is modulo the specified width, preventing out-of-range overflow.

- Prefer ap_fixed<W,I> or ap_ufixed<W,I> over floating-point for efficient fractional math.

- Specify total bit width (W) and integer bits (I) to balance range and precision.

- Use fixed-point to reduce hardware cost while meeting numeric accuracy requirements.

- Convert constants and variables to ap_fixed where feasible (e.g., coefficients, accumulators in filters).

- Handle scaling and overflow carefully; HLS manages binary point arithmetic automatically.

- Choose the smallest data type that fits the algorithm’s range requirements.

- Use ap_uint<1> for boolean flags and narrow ap_int/ap_uint types for small ranges instead of default 32-bit int.

- Prefer exact-width C types (uint8_t, uint16_t, etc.) or ap_int for non-standard sizes.

- Smaller data types save FPGA resources and can reduce latency (smaller adders and multipliers).

- Consider enumerated types for states or small value sets; HLS minimizes their bit width automatically.

- Declare read-only tables as const (and static if inside a function) so HLS infers them as ROMs.

- Use const values to enable constant propagation and let the tool optimize calculations.

- Static const arrays are implemented as ROM (LUTs or block RAM) without write logic.

- Always initialize static variables explicitly to avoid ambiguity, even though they default to zero.

- Use C/C++ structs to group related signals (e.g., RGB pixel or complex number).

- By default, top-level structs are aggregated into a single wide bus; internal arrays of structs are disaggregated into parallel arrays.

- Use #pragma HLS AGGREGATE or DISAGGREGATE to override default mapping when needed.

- Aggregate structs if you want them treated as one chunk of bits (e.g., one memory), or disaggregate to expose fields as separate ports.

- Apply structs to improve readability and ensure related signals travel together.

- Use float or double only when the algorithm truly requires IEEE-754 dynamic range or library math functions.

- Be aware that floating-point operations consume significantly more FPGA resources and have higher latency than fixed-point.

- Floating-point IP cores are pipelined (II=1 for add, mult, etc.) but results may differ slightly from software due to rounding and associativity.

- Limit floating-point usage to where necessary; prefer fixed-point for most internal calculations and convert only at interfaces if required.

- Account for conversion costs when mixing floating-point and fixed-point types.

- Do not use malloc, free, or any dynamic memory allocation in HLS; hardware has no heap.

- Avoid variable-length arrays and dynamic containers (std::vector, std::string, etc.).

- All arrays must have fixed compile-time bounds.

- If variable size is required, declare the maximum size and use loop limits to control usage.

- Do not overuse the volatile keyword in HLS code.

- Use volatile only for memory-mapped I/O or hardware/software status flags that truly require it.

- Avoid marking arrays or normal variables as volatile; it disables key optimizations like burst combining and dead code removal.

- Unnecessary volatile usage can severely reduce performance and block memory optimizations.

- Do not rely on implicit type conversions that lose precision in HLS code.

- Be explicit when narrowing types; always cast intentionally to smaller widths (e.g., int → ap_int<8>).

- Avoid mixing signed and unsigned types without care; it can generate unexpected comparison logic.

- Use consistent unsigned types for loop and array indices to prevent mismatched hardware.

- Do not use double if float suffices; halving bit-width reduces resources and latency.

- Do not use unions for type-punning; HLS does not support synthesizing them for bit reinterpretation.

- Use ap_uint of equivalent width with manual bit slicing, or HLS reinterpret cast, for safe bit-level casting.

- Avoid std::complex; define your own struct with separate real and imaginary fields.

- Do not use multi-level pointers (e.g., pointer-to-pointer) as top-level function ports; HLS does not support them.

- Do not perform arbitrary pointer arithmetic on top-level memory interfaces (AXI4).

- Sequential pointer increments or array-style indexing are safe and allow burst transfers.

- Non-sequential pointer jumps break burst inference and cause inefficient single transactions.

- For random access, pass data as an array with ap_memory interface instead of using pointer arithmetic.

- Do not cast interface pointers to different types (e.g., int* to char*); it may force byte-wise accesses or fail synthesis.

- Do not use pointer-to-pointer types (e.g., type**) as top-level function ports; HLS cannot synthesize them.

- Avoid arrays of pointers unless they reference fixed, known global objects.

- Replace arrays of pointers with 2D arrays or flat arrays for synthesizable memory access.

- Keep memory access patterns simple and regular so HLS can infer efficient memory architectures.

# Structural Design

- Treat C++ code as a hardware description; write with hardware structures in mind.

- Use if/else and switch to describe conditional logic that maps to multiplexers.

- Use loops to describe iterative hardware like shift registers or accumulators.

- Model FSMs with enums for states and a switch-case for transitions, updating the state each iteration.

- Use static variables to represent registers or storage that must retain values across cycles.

- Use static local variables or globals to model registers or memories that persist across function calls.

- Initialize static variables explicitly; uninitialized statics default to zero on reset.

- Declare arrays as static for shift registers or delay lines so they retain values across calls.

- Each function instance shares its static variables; use templates or unique functions if separate copies are required.

- Use C++ templates to build parameterized and reusable hardware blocks.

- Each unique template parameter set generates a separate hardware instance.

- Pass constants such as bit-width, depth, or IDs as template parameters to control hardware size and uniqueness.

- Templates help avoid code duplication while instantiating distinct logic (e.g., shift registers of different sizes).

- Ensure all template code is synthesizable and resolves to a finite, unrolled design.

- For pure combinational logic, compute outputs only from inputs; do not use static variables or memory.

- For sequential logic, use static state to hold values across iterations or function calls.

- Model sequential behavior with a static state variable, a switch/if block for transitions, and state updates each cycle.

- Use loops or function calls to represent multi-cycle behavior, ensuring loop bounds reflect intended cycle counts.

- Label loops when needed to clarify structure and improve readability.

- Write C/C++ in a synthesis-friendly style that HLS can easily analyze and optimize.

- Use simple for loops with clear, compile-time bounds; avoid recursion and deep call stacks.

- Use bitwise operators (&, |, ^, <<, >>) for bit manipulations; all are synthesizable.

- Prefer compile-time constants or enums for array bounds and switch cases.

- Apply #pragma HLS UNROLL or PIPELINE to innermost loops to control replication or pipelining.

- Ensure memory accesses have predictable indexing patterns; avoid irregular or unpredictable indices.

- For random indexing, keep patterns regular or use dual-port memories for parallel reads/writes.

- Always verify HLS designs at the C/C++ level before synthesis.

- Write a testbench that feeds sample or random inputs and checks outputs.

- Use C simulation to catch issues such as uninitialized variables, array out-of-range, or excessive combinational logic.

- Validate that the design produces correct outputs and meets expected latency/throughput behavior.

- Do not use recursion; hardware cannot synthesize functions with unbounded call depth.

- Avoid unbounded while-loops with unpredictable exit conditions.

- If variable loop bounds are required, add safety limits or apply #pragma HLS LOOP_TRIPCOUNT.

- Ensure all functions and loops resolve to finite, predictable hardware.

- Do not use OS features in synthesizable code; FPGA fabric has no operating system.

- Remove or guard file I/O (fopen, printf), system calls, threads, and mutexes from synthesis paths.

- Wrap debug-only code with #ifndef __SYNTHESIS__ so it runs in simulation but is ignored in synthesis.

- Do not use standard library containers (vector, list, string, etc.) that require dynamic memory.

- Use plain arrays, ap_int/ap_fixed, and structs to ensure synthesizable hardware mapping.

- Do not assume C/C++ simulation behavior matches hardware implementation.

- Always initialize variables before use; uninitialized values in hardware are undefined.

- Avoid shifting or indexing beyond array bounds; results in hardware are incorrect or undefined.

- Do not use function pointers or virtual functions; HLS requires deterministic hardware binding.

- Avoid excessive use of global or static variables in HLS designs.

- Each global variable becomes a persistent hardware resource that is always present.

- Multiple writers to the same global create dependencies and may serialize access.

- Prefer passing values through function arguments or interfaces instead of globals.

- Reserve globals for true constants or carefully managed shared state.

- Do not neglect reset behavior; always define initial values for registers and state.

- HLS resets statics and globals to default (zero) unless explicitly initialized.

- If non-zero or specific reset values are required, code them directly at declaration.

- For shift registers or static arrays, preload with zeros or known patterns as needed.

- With ap_ctrl_hs, the block restarts fresh each invocation; with ap_ctrl_none, add explicit reset logic if required.

# Control I/O Handling

- Use AXI4-Lite (s_axilite) for all scalar control parameters and configuration registers.

- Apply #pragma HLS INTERFACE s_axilite port=<variable> bundle=control for each scalar, including a return port.

- Ensure exactly one port is marked port=return in the control bundle to manage start/stop/status.

- Expose loop bounds, offsets, and algorithm modes as s_axilite ports for host configuration.

- Follow this convention so the generated IP integrates easily with software control.

- Read s_axilite control parameters once at the beginning of each invocation.

- Store control values (e.g., threshold) in local variables and reuse them instead of re-reading the interface.

- Pass control values as arguments to sub-functions if needed in multiple places.

- If a control is constant across all runs, consider making it a template parameter or #define.

- If control values must change during runtime or align with data, send them through a streaming interface.

- Use an hls::stream of control tokens that travel in parallel with the data stream.

- Duplicate or fork control tokens if multiple modules require the same value.

- Forward control values downstream to keep modules working in lockstep with the same configuration.

- Do not read the same control parameter independently in multiple parallel processes.

- Read the control once, store it in a temp, and distribute it to all processes that need it.

- Duplicate the control into multiple FIFOs or use a distributor function in dataflow to fan out values.

- Ensure all processes use the identical control value for the same invocation to avoid mismatch.

- Use ap_ctrl_none on the top function only for always-running dataflow engines.

- In ap_ctrl_none mode, the IP starts automatically after programming and runs continuously.

- Control registers (s_axilite) are still valid, but there is no done signal or automatic restart.

- Manage lifecycle externally (reset/disable as needed) and handle control register updates carefully.

- Apply this mode only for continuous streaming or static pipeline applications.

- Do not assume AXI4-Lite control changes apply immediately during computation.

- If a register is read once at start, updates are not visible until the next run.

- Continuous reads may see updates, but mid-run changes can break assumptions and cause mismatches.

- Only change control registers safely between accelerator runs, or apply reset before using new values.

- Do not treat control registers as real-time adjustable unless the hardware FSM is explicitly designed to handle it.

- Do not break one dataflow pipeline on a scalar/global control while assuming others stop too.

- If parallel processes use a shared stop condition, ensure they all receive it in lockstep.

- Avoid using global controls as direct break conditions inside parallel loops; this can cause deadlock or starvation.

- Use handshake or stream-based control distribution to synchronize stops across all parallel processes.

- Handle global stop conditions before entering the dataflow region, or partition input based on control.

- Do not use static or global variables as shared control flags across functions.

- A global in hardware is a physical register with no handshake; using it for control causes race conditions.

- Only write to a global in one place if absolutely needed, and use it synchronously elsewhere.

- Prefer passing control values through arguments or streams for explicit synchronization.

- Remember that global scalars are not external interfaces unless declared as such with pragmas.

- Always test accelerator behavior under different control parameter values (sizes, modes, thresholds).

- Validate edge cases such as size=0 or maximum bounds to uncover hidden issues or deadlocks.

- Check hardware timing and resource usage for all control values; larger bounds may affect II or memory.

- Ensure control-dependent code paths are verified both in C simulation and on FPGA.

- Do not overlook backpressure handling on streamed control signals.

- Each module must read exactly one control token per data unit (or per defined protocol).

- If a module fails to read, the control FIFO will stall; if it over-reads, it will wait forever.

- Treat control streams with the same push/pop discipline as data streams.

- Ensure downstream blocks consume each control token once; use pass-through or forwarding if multiple modules need it.