- Apply #pragma HLS DATAFLOW at the top scope

- Split input/compute/output into separate tasks/functions

- Connect tasks only via hls::stream or ping-pong buffers (no shared globals/arrays)

- Assign #pragma HLS STREAM depth>=1 for every stream; size to cover consumer latency/bursts

- Insert a FIFO (depth>=1) on any feedback path to break zero-latency cycles

- Pipeline inside each task with #pragma HLS PIPELINE to sustain overlap

- For true parallel hardware, do not inline or reuse the same function instance across tasks

- Apply #pragma HLS PIPELINE to innermost or performance-critical loops

- Always target II=1; accept higher II only as an intentional trade-off

- Resolve loop-carried dependencies by reorganizing code or buffering

- Use #pragma HLS DEPENDENCE false only when dependency is truly absent

- For streaming/infinite loops, use rewind; add enable_flush/free_running only if needed

- Prefer loop-level pipelining; use function-level pipeline only for continuous streaming kernels

- Combine pipeline with loop transformations (unroll, flatten, merge) when beneficial

- Unroll small loops partially to expose parallelism; ensure enough memory ports with #pragma HLS ARRAY_PARTITION

- Use #pragma HLS LOOP_TRIPCOUNT with sensible min/max for non-constant loops

- Avoid multiple loop exits; restructure to a single exit when possible

- Do not pipeline loops with recursion or unbounded termination

- Prevent memory/operator conflicts by partitioning arrays or increasing resources

- Never ignore II > 1; identify and fix the cause (dependencies, memory ports, resources)

- Do not fully unroll large loops unless resources and timing allow

- Prefer pipelining with II=1 over full unrolling for resource efficiency

- Partition design into multiple functions, each handling one stage or kernel

- Keep one primary loop per function and optimize it in isolation

- Use #pragma HLS DATAFLOW on independent sub-functions to run in parallel

- Connect sub-functions with hls::stream (with depth>=1) or arrays/pointers

- Apply proper interface pragmas: m_axi for memory, axis for streams, s_axilite for config, ap_ctrl_hs at top

- Use #pragma HLS INLINE for small utility functions or single-use helpers

- Use INLINE off for functions that must remain distinct hardware blocks

- Do not inline functions intended for parallel execution under DATAFLOW

- Each unique function or template generates one hardware block; duplicate or template code for parallel instances

- Use ap_ctrl_none only for internal free-running sub-functions inside dataflow

- Balance latency between parallel branches with FIFO depth to prevent stalls

- Avoid global or static variables across functions; pass data via arguments or streams

- For feedback paths, insert FIFO depth>=1 and ensure initialization to avoid deadlock

- Do not implement entire design as one monolithic function; modularize into logical stages

- Avoid unsynthesizable constructs: no malloc/new/free/delete, recursion, or virtual functions

- Prefer ap_int<W>/ap_uint<W> with exact widths over native int/long

- Match bit width to range; use smallest safe width, add bits if overflow possible

- Prefer ap_fixed<W,I>/ap_ufixed<W,I> over floating-point for fractional math

- Use float/double only when IEEE-754 dynamic range is strictly required

- Convert constants and coefficients to fixed-point when feasible

- Use ap_uint<1> for flags; prefer narrow types instead of default 32-bit int

- Use enums for states; HLS auto-minimizes their width

- Declare const/static const arrays as ROM so HLS infers LUT/BRAM

- Group related signals in structs; use AGGREGATE/DISAGGREGATE to control port mapping

- Avoid dynamic memory (malloc/free, std::vector/string); all arrays need fixed compile-time bounds

- If variable size needed, declare max size and control with loop limits

- Avoid volatile except for true memory-mapped I/O or status flags

- Be explicit in casts; avoid implicit narrowing and signed/unsigned mismatches

- Do not use unions or std::complex; use ap_uint with bit slicing or custom structs

- Keep top-level ports simple: no pointer-to-pointer, no arrays of pointers, no type-casting of interfaces

- Treat C/C++ as hardware description; if/else/switch map to muxes, loops map to hardware structures

- Model FSMs with enums for states and switch-case for transitions; update state each cycle

- Use static variables or arrays to represent registers, shift registers, or memories across cycles

- Each function instance shares its static variables; use templates or unique functions if separate copies are required

- Use C++ templates to build parameterized reusable blocks; each parameter set creates unique hardware

- For combinational logic, compute outputs only from inputs; avoid static state

- For sequential logic, hold state in static variables and update each cycle

- Avoid recursion, dynamic memory, OS calls, and standard containers; use arrays/ap_int/ap_fixed/structs

- Ensure all loops have finite bounds; apply #pragma HLS LOOP_TRIPCOUNT for variable loops

- Prefer compile-time constants or enums for array sizes and switch cases

- Use #pragma HLS UNROLL or PIPELINE pragmas on innermost loops to control concurrency

- Keep memory access patterns regular; use dual-port or partitioning for parallel access

- Initialize all variables and statics explicitly; undefined values cause mismatches

- Do not rely on global variables except for constants or carefully managed shared state

- Always code explicit reset behavior for registers and state if non-zero initialization is required

- Use AXI4-Lite (s_axilite) for all scalar control parameters and include one port=return in the bundle

- Read s_axilite parameters once at start of each run; store in locals and reuse

- Pass control values as arguments or distribute via streams, not as shared globals

- For runtime-varying controls, send them through hls::stream tokens in parallel with data

- Distribute or fork control tokens so all processes get the same value in lockstep

- Do not reread the same control in multiple parallel tasks; read once and fan out

- Apply ap_ctrl_none only for always-running streaming accelerators

- Update control registers only between runs; mid-run changes are unsafe unless explicitly handled

- Synchronize global stop/enable conditions using handshake or stream tokens, not globals

- Avoid using static or global variables as control flags across functions

- Always specify interface pragmas: s_axilite for scalars, axis for streams, m_axi for memory

- Treat control streams with strict push/pop discipline; each token consumed exactly once per protocol

- Add FIFO depth when distributing controls to parallel branches to avoid stalls

- Test accelerator behavior under different control values and edge cases

- Validate control-dependent code paths in both C simulation and FPGA